Introduction

In 2023 Allianz published a report about the challenges of offshore wind farm projects.

At the time we are writing this book exciting events are happening in the industry. European and Chinese OEMs are battling to win supply contracts. Vestas and GE indicated that their focus is on reliability and not on the size. They are concentrated on the 15MW platform. On the other hand SGRE announced the development of a 21 MW wind turbine. Chinese OEMs installed already an 18 MW machine and are the preferred supplier for a project in German waters.

Sebastian works for more than 15 years in the offshore wind industry in the risk ans insurance management.

Augusto is amechanical engineer who started to work in the risk and insurance management for a energy supplier.

Our motivation is to clarify the reader the different an nuanced aspects of the risk and insurance management of offshore wind farms. There is almost no literature about this topic. Some important work has been written for the oil and gas industry by David W. Sharp.

We have seen that team members of the technical teams have various misconceptions about insurance topics or simply did not know how the insurance program including the Construction All Risk Policy works. That is absolutely not their fault. Insurance is a complex topic, which a simply word can change the cover. For us has not been easy, as well. We had to read and re-read contract clauses, discuss internally and with lawyers and brokers. Sometimes having not a straight conclusion.

We want to bring the reader the experience collected.

We want to take you through an exciting world of risk and insurance management of offshore wind farms.

We have been lucky to be part of an incredible journey through the world of risk and insurance management of offshore wind farms. Development, construction, and operations phases.

The costs of insurance are relatively low in comparison with the rest of the lots e.g. the supply of the Wind Turbines or Foundations, however the consequences of a not comprehensive insurance cover could costs the project developer expensive.

We will bring you with us through an exciting journey, where we share with you our experiences and we hope you will learn more about insurance and risk management.

To make it more fun, in this book, we will bring you with us as an insurance and risk management team member. Our objective with this book is to bring you as close as possible to our daily work, and we hope, that as it is for us, that you enjoy it and probably learn something.

Hypothetical Scenario

To reach our objective, we have chose an hypothetical project, which has already an area in the economical zone. That means, that auction to win the area is not part of this scenario.

The capacity of the project is 2 GW which will comprise of 15 MW X Wind Turbine Generators (WTGs), foundations including transition piece less monopiles, inter array cables, an offshore HVDC converter station, export cable, an onshore substation and a onshore control room.

The annual production of the OWF is X GWh, with a total revenue of X EUR.

At the time of writing this book, 15MW WTG are already being constructed in project in Europe and are entering into serial production. Vestas received the Type Certificate in XX day (include reference) and SGRE on this date.

The OWF is close enough to other OWF, in which if needed personnel can sleep over in the Offshore Subtation Platform.

Close the OWF in Construction a Gas Pipeline is located.

In addition, the Russia war against Ukraine is ongoing and does not seem to have an end.

12.07.2024 Nato has accused China to be enabler of the war against Ucraine. We are mentioned this, since this is part of Geopolitical risks.

russian ships in Neatherlands offshore wind farm.

Insurance

Insurance as a concept has ancient origins, dating back to early human societies where mutual aid and communal support systems were common. The earliest known insurance practices can be traced to the Code of Hammurabi in Babylon around 1750 BC, which included provisions for merchants to receive loans to finance shipments with the understanding that the lender would cancel the loan if the shipment was lost or stolen. In ancient China, merchants spreading their cargo across multiple ships to reduce the risk of loss represents another form of early risk management. Similarly, in ancient Greece and Rome, maritime loans functioned as a form of insurance for sea voyages, where lenders absorbed the losses if ships were lost at sea. These early practices laid the groundwork for more formalized insurance systems, which evolved significantly in the 17th century with the establishment of the first insurance company, Lloyd's of London, which became synonymous with marine insurance. This marked the beginning of modern insurance practices, providing structured financial protection against various risks and uncertainties.

The law of big numbers.

The origins of insurance in the offshore wind industry is based in the oil and gas sector. The insurance market has played a crucial role in the development of the oil and gas industry, as it is now also in the offshore wind industry, providing essential risk transfer solutions and financial protection for companies, shareholders, and the environment[2].

As the exploration and extraction of oil and gas involved significant financial investments and substantial risks, companies sought ways to mitigate potential losses. The development of specialized insurance products, such as Construction All Risk (CAR) and Operational All Risk (OAR) insurance, became essential. CAR insurance emerged to cover risks during the construction phase, protecting against physical damage and loss due to unforeseen events like natural disasters or accidents. Delay in Start-Up (DSU) is a specialized cover designed to mitigate the financial losses resulting from delays in project completion due to physical damage or other insured events during the construction or installation phase. OAR insurance covers operational risks once the projects are up and running, including equipment breakdowns, production interruptions, and property damage.

Marine insurance also plays a critical role in the oil and gas sector, covering vessels used for the transportation of equipment and personnel, as well as those involved in maintenance and support operations. Marine insurance policies were developed to protect against the loss or damage of ships, cargo, and liabilities during transit. Lloyd's of London played a significant role in pioneering marine insurance for the oil and gas industry, offering coverage for tankers and other vessels. For the vessels involved in the construction and operation of oil and gas projects, specific coverages like Hull and Machinery (H&M) insurance protect against physical damage, while Protection and Indemnity (P&I) insurance covers third-party liabilities.

Like their counterparts in oil and gas, offshore wind projects involve significant investments and face substantial risks, necessitating robust insurance solutions. The offshore wind industry has adopted similar insurance programs to those used in the oil and gas sector. Offshore wind projects utilize Construction All Risk insurance,

DSU, additionally, specialized coverages for vessels involved in offshore wind construction, such as installation vessels, mirror those used in the oil and gas industry. Once operational, wind farms typically employ Operational All Risk policies to protect against physical damage and business interruption. In the O&M phase, the vessels used have as well H&M and P&I insurance cover.

One additional common topic between oil and gas and offshore wind industry is the use of floating platforms. offshore wind floating wind turbines technology is similar and in some cases based on technology such as the semi-submersible rigs used in oil and gas industry. Both industries face significant risks, including harsh weather conditions and logistical challenges. Insurers are looking very closely to what problems are facing the current installed prototypes and relatively small floating offshore wind farms. They are prototypical. And that is something an Underwriter is carefully with. There are discussions about the design and the standardization of the floating platforms. Some critical components as the Inter Array Cables, use a special technology know as Dynamic Power Cables, since the platforms are not fixed in the bottom of the seabed but move around. Insurers are agreeing to grant London Engineering Group (LEG) 2 Design Exclusion, which limits the cover provided by the insurance. In addition, logistics including ports availability affects how the insurers perceive risks related to business interruption for example in the case of a Major Component Exchange.

As the offshore wind fix-bottom sector already has some critical mass, insurers are continuously learning and adapting. The insurance programs mentioned earlier are not set in stone; they are in a state of constant evolution to address emerging challenges. Insurers sometimes reduce coverage to the detriment of project owners, introducing exclusions such as the Cable Protection System (CPS) Exclusion JRC XX, motivated by losses in various offshore wind farms in 2021. But the insurers are not just the ones who learn and adapt. Project owners and contractors have learned from CPS damage, implementing better designs and mitigation measures. Consequently, some underwriters have started to reconsider and not exclude CPS in the CAR and OAR policies, or to soften the wording used.

One such challenge is the introduction of bigger turbines such as SGRE 22 MW turbines, which could significantly impact property damage and business interruption risks. Bigger turbines will need to use bigger vessels, which requires higher costs for mobilization and demobilization. Additionally, supply chain bottlenecks, driven by high demand for critical components such as cables, transformers, switchgears, pose substantial risks in case of loss or damage, affecting the business interruption or DSU cover.

Definitions

In the previous introductory section about insurance and offshore wind, we have included already few words and abbreviations. If you work in the insurance industry as broker or underwriter, these terms are familiar to you, however, if you are part of the technical team and have occasionally contact with your insurance colleagues, all the different terms can be confused. That is why, here, we have prepared a list of terms that we will use in this book.

Additional Insured: A person or entity added to an insurance policy, providing them coverage under the primary insured's policy terms.

Basis of Recovery: The method by which losses are calculated and compensated under an insurance policy, such as replacement cost or actual cash value.

Business Interruption (BI): Insurance covering the loss of income due to a halt in operations caused by insured perils.

Buy-Back Exclusion: An option allowing policyholders to reinstate coverage for specific excluded risks, often for an additional premium.

BIMCO:

Captive:

Claims: Requests made by the insured to the insurer for payment of benefits under the policy terms following a loss or damage.

Contingent Business Interruption: Coverage for lost income due to disruptions in the supply chain or operations of key suppliers or customers.

Contractor´s group:

Deductible: The amount the insured must pay out-of-pocket before the insurance coverage begins to pay for a loss.

Defects Notification Period:

Delay in Start-Up (DSU): Coverage for financial losses due to delays in project completion, typically due to insured physical damage.

EML (Estimated Maximum Loss): The maximum anticipated loss that could occur from a single event, used for risk assessment and determining coverage limits.

Employer Group:

Exclusions: Specific conditions or circumstances listed in the policy that are not covered by the insurance.

Indemnity: A principle where the insurer compensates the insured to the extent of their loss, restoring them to their pre-loss financial position.

Indemnity Period: The duration during which benefits are payable following a loss, often related to business interruption coverage.

Insured Peril: A specific risk or cause of loss covered by an insurance policy, such as fire, theft, or natural disasters.

Liability: Legal responsibility for damages or loss, which can be covered under various liability insurance policies.

Limits: The maximum amount an insurer will pay for a covered loss, as stated in the policy.

Loss or Damage: Physical injury, destruction, or loss of property covered under an insurance policy.

Loss Ratio:

Marshalling harbour:

Negligence-Based Liability: Liability arising from the failure to exercise reasonable care, resulting in damage or injury to another party.

Occurrence: An event that triggers coverage under an insurance policy, often defined as an accident, including continuous or repeated exposure to substantially the same harmful conditions e.g. storm. For the latter will be in the Insurance Policy a defined amount of time for such occurrance, normally 72 hours.

Premium: The amount paid by the insured to the insurer in exchange for coverage under the insurance policy.

Property Damage: Physical damage to or destruction of tangible property, covered under property insurance policies.

Protective Co-insurance:

Sites: Site 1: manufacturing site, site 2: marshalling harbour, site 3: offshore construction site.

Third-Party: An individual or entity who is not directly involved in an insurance contract but may be affected by a claim, typically a person who claims to have been harmed by the insured's actions.

Waiting Period: The period of time that must pass before benefits are payable after a claim is filed or a loss occurs.

Waiver of Subrogation: A provision where the insurer waives its right to pursue recovery from a third party responsible for a loss after compensating the insured.

Wording (Policy Wording): The specific language and terms used in an insurance policy that define the coverage, exclusions, conditions, and limitations.

https://www.lexology.com/library/detail.aspx?g=d0a8430e-9e76-4f63-97b4-515afe3992ce

Who are the Stakeholders of the insurance team

The success of offshore wind projects hinges on the collaboration of various stakeholders, each playing a critical role. This a non exhaustive list of stakeholders, which you will be interacting with, either directly or indirectly.

Broker: Acts as an intermediary between the project stakeholders and the insurance market, helping to secure appropriate coverage and negotiate terms.

Claims adjusters and loss prevention specialists, play vital roles in managing risks and handling incidents.

Contractors and Subcontractors: Execute the construction, installation, and maintenance work required for the project, often involving specialized skills and equipment.

Contractor insurance team:

Contract Managers:

Financial modellers: they are interested in the insurance premiums, cover and financialrisk mitigation measures which the insurer provides. This information will flow in the CAPEX.

Financial controllers:

Force Majeure:

Lenders: Provide the necessary financing for the project, assessing risks and returns to ensure the viability and profitability of their investment.

Lenders Insurance Advisor:

Liquidated damages:

Loss of use:

Project Directors: Oversee the overall planning, execution, and management of the offshore wind project, ensuring it meets its objectives.

Risk managers

Underwriters: They are part of the insurer provider team. Evaluate the risks associated with the project and provide insurance coverage, determining premiums and policy terms.

Transmission System Operator (TSO): Manages the electricity transmission network, ensuring the generated power from the offshore wind farm is efficiently integrated into the grid.

State Agencies: Regulate and oversee compliance with environmental, safety, and operational standards, and may provide permits and approvals necessary for project development.

Shareholders: Individuals or entities that own shares in the offshore wind park, sharing in its profits and having a stake in its success. Shareholders can include a mix of private companies, public entities, and individual investors, each contributing to the capital required for the project and having a vested interest in its financial performance.

Take over:

Warranty period:

Wilful misconduct:

Work Package:

Work Package Managers are your colleagues within the project. They are responsible for overseeing specific components or phases of the project. They will normally lead the tender process and after that the execution of their part of the project.

Supply Contracts

To enable the construction on the seabed, Geotechnical and Geophysical Surveys and evaluations of the results will be contracted. This information is then used to design the foundations and can be used also in a later stage for the installation of the foundations.

To put in operation an OWF, the Project SPV must conclude tens of contracts being the most important:

- WTG Supply, also known as Turbine Supply Agreement (TSA), SPA

- WTG Transport and Installation (T&I)

- WTG Service and Maintenance Agreement (SMA)

- FOU Monopiles (FOU-MP)

- FOU T&I

- Inter Array Cable (IAC) EPCI (preferred)

- Offshore converter station

- Offshore converter station T&I

- Export Cable EPCI (preferred)

- Onshore Export Cable EPCI

- Marine Warranty Surveyor

Marine Construction and Coordination

- Trasport of equipment and personnel: CTV, Helicopters, SOV.

- Guard Vessel

- Communications systems, which could be installed in a neighbouring wind park.

For the operation,

In each of these contracts, there is an insurance and a liability clause. In most cases, our colleagues reach out to ask to have a look at the insurance requirements. However, to be able to set requirements regarding insurance, we need to understand the liability and consequential damage clause.

Insurance Strategy

Knowing that we will be part of the team that will construct this 2 GW Offshore Wind Farm in the North Sea, we have to

For the insurance strategy we need to consider the timelines. From the point the Project receive the authorization from the State to construct the wind farm, it will pass 5 to 7 years until the Project finishes the construction phase and starts the operation phase. The insurance team is busy however from the beginning.

Since the Project

Construction All Risk (CAR)

- Serial losses

- Offshore cancellation

- Extended Maintenance

- Multi insured clausel

Vessels: Installation Vessel (floating and jack up Vessel)

Cargo

Operation All Risk (OAR)

Third party liability

Strict Liability

Consequential losses

H&M

P&I

LEG Design Exclusions 1, 2, 3

WTG

- Serial defects

WTG SMA

- Full Service Contract

- No Employers Insurance requirements

- What liability regime: in HD knock-for-knock

- Availability Warranty

- MM: Damage to

Contract review

When reviewing the contract with the insurance lenses, we have to

- Under definitions: Employers Group and Contractors Group.

- Liability clause

- Limit of liability clause

- Consequential Damage Clause

- Insurance

In the Insurance which can be part of the contract or an annex to the contract we will have normally two parts: Employers Insurance and Contractors Insurance.

Claims

Collision of two vessels outside of the construction field

Incident Overview:

A third-party Vessel collided with the port side bow entrance of the transporting barge while the barge was transporting three monopiles and two transition pieces. The collision resulted in damage to both the barge and the surface damage to the monopile. No person was injured in the accident, and there is no oil spilled out of the barge, since the barge was being tugged by tugboats. This happened on the way to the construction field offshore.

In this incident, we have four main aspects to consider from the insurance perspective:

Barge, Monopiles and Transition Pieces, Personnel and Environment. We will analyse one by one.

- Barge:

Following the incident, the barge could navigate back to the marshalling harbour, and it was safely moored in the harbour. An initial inspection was conducted on the barge's hull, revealing the extent of the damage. Divers are scheduled to assess the barge's underside for further damage.

T&I Contractor Insurances:

Since the CAR Insurance does not cover any Vessel related claim, the T&I Contract will submit an insurance claim to their broker for the damage caused to their barge.

- Hull and Machinery (H&M): Covers physical damage to the barge. The T&I contractor submitted a claim to their insurer, who assigned a damage surveyor to assess the extent of the barge’s damage. The T&I Contractor believes the third-party Vessel is at fault and plans to subrogate the claim to the vessel owner's insurer.

- P&I: This is a third-party liability insurance. This insurance will kick in, if it is found, that the Contractor caused damage to the other Vessel, in excess of the H&M cover.

- Monopiles and Transition Pieces

In the first initial inspections indicate only surface-level damage to the monopile, a thorough examination is necessary to rule out any structural impact.

- CAR Insurance: Meanwhile, the Project contacted their insurance broker, and filed a claim with the Insurer for the potential damage to the monopile. Awaiting a detailed damage report, the Insurer has not yet commissioned additional technical experts.

Both the T&I Contractor and the Project have taken steps to assess the damage and initiate the insurance claim process for their respective assets. Further actions, including repairs and potential litigation, will likely depend on the findings of the ongoing damage surveys and investigations.

Open points: consequential damages.

Offloading the cargo: The immediate priority is to transfer the monopile from the damaged barge back onto the quayside of the terminal. After a comprehensive assessment is done, the barge will be freed up for transport to a shipyard for repairs.

Thorough inspections: Divers will conduct an underwater inspection of the H-407's hull to assess the full extent of the damage.

Concurrently, experts from MWS will perform a detailed assessment of the monopile. This inspection may include surface analysis and potentially phased array ultrasonic testing to check for internal damage.

Information Gathering: the Project will gather information from both MWS and the Contractor, including inspection reports and damage assessments.

The Contractor will continue to gather evidence to support their claim against the third-party Vessel, including witness statements, data from the towboat data recorder, and calculations regarding the impact.

Collaboration and Reporting: the Project will share relevant information, including the damage report from the MWS, with the monopile's designer, project certifier, and their Insurer.

Consultation with insurers: Both the Project and the Contractor will maintain close communication with their respective insurers throughout the process. All repair costs and proposed actions will be discussed with the insurers before proceeding.

The Project and Contractor will need to determine which insurance policy will cover the indirect costs associated with the incident, such as project delays. While ENBW believes their CAR policy would cover any costs not covered by other policies, they acknowledge they need to confirm the specific language in the policy.

Construction and Operation

For packages like WTG and OSS there is

Inter Array Cables

Inter-array cables connect individual turbines to each other and to the offshore substation. These cables enable the transmission of electricity generated by the turbines, ensuring it reaches the offshore substation for consolidation before being sent to the onshore grid.

Subsea Cable Structure:

- Conductors: Typically made of copper or aluminium, these conductors are the core of the cable, responsible for carrying electrical current.

- Insulation: Surrounding the conductors, insulation materials prevent electrical leakage and ensure safe operation.

- Armor: Protective layers of steel or other materials provide mechanical protection against physical damage.

- Optical Fibers: Often included for communication purposes, allowing for data transfer and control signals.

Pre-Installation Surveys:

Seabed Surveys: Comprehensive surveys to determine the optimal cable route, identifying obstacles and areas of instability.

Seabed Conditions: Assessment of conditions to ensure the chosen route is feasible and safe for cable laying.

Cable Laying:

Deployment: Specialized vessels equipped with dynamic positioning systems lay the cables precisely along the planned route.

Real-Time Monitoring: Constant monitoring and adjustments are made to manage tension and prevent damage during laying.

Cable Burial:

Techniques: Post-lay burial using jetting or plowing to protect cables from external hazards.

Burial Depths: Careful calculation of burial depths to balance protection and installation feasibility.

Cable Protection Systems (CPS)

The purpose of the CPS is to protect the cables at critical points where they transition from the seabed to the turbine or substation foundation. These systems mitigate risks from mechanical damage, seabed movement, and marine life interactions.

Components:

- Bend Stiffeners: Prevent excessive bending at the cable’s entry points.

- Bend Restrictors: Limit the curvature of the cable to prevent damage.

- Protection Covers: Shield the cables from abrasion and impacts.

Stabilization of the CPS:

Rock bag berms are an effective method for stabilizing Cable Protection Systems (CPS). These berms consist of rock-filled bags strategically placed to provide additional weight and stability to the CPS, preventing movement and abrasion caused by environmental forces such as waves and currents. The primary function of rock bag berms is to create a stable base that can withstand orbital wave velocities and current-induced forces, thereby protecting the CPS from dynamic environmental conditions.

In 2021 Ørsted faced significant challenges with Cable Protection Systems (CPS) in several of its offshore wind farms, leading to substantial insurance claims and financial repercussions. Not just the operator and owner of the project were under pressure, but also the CPS Manufacturer. The problem was first detected during an inspection at the Race Bank wind farm and has since been observed in up to ten wind farms, potentially costing up to EUR 403 million in repairs and mitigation measures.

The root cause of these issues has been identified as inadequate scour protection, which left the inter-array cables unstabilized. This lack of stabilization allowed the CPS to move across the scour protection rocks, causing abrasion and, in some cases, cable failure.  In order, to stabilize the CPS permanent rock or rock bag berms was planned to be placed on the existing scour protection.

#### Risks During Installation

The successful deployment of a subsea cable system hinges on a careful installation process. However, several risks can arise during this critical phase, potentially causing delays, damage, and financial setbacks. This chapter explores the key threats to a smooth installation and the importance of considering these factors during the planning and execution stages.

Mechanical Damage

Several external factors can inflict physical damage on subsea cables:

Vessel Anchors, Fishing Gear, and Seabed Debris: Anchors dragging across the seabed, fishing nets, and other debris pose a significant threat to cable integrity. Careful route planning and pre-lay surveys are crucial to mitigate these risks.

Handling Errors: Improper handling during transportation, loading, and deployment can cause kinks or bends in the cable, compromising its long-term functionality. Stringent training for personnel involved in cable handling is essential.

Environmental Factors

The unforgiving nature of the ocean environment presents its own set of challenges:

Adverse Conditions: Rough weather, strong currents, and unpredictable wave patterns can disrupt the installation schedule, leading to delays and potentially causing damage to the cable or equipment. Close monitoring of weather forecasts and contingency plans are necessary.

Seabed Instability: Shifting seabed formations or uneven terrain can complicate the burial process, resulting in cables being left inadequately protected or unevenly buried. Pre-lay surveys and geotechnical assessments can help identify potential challenges.

Operational Challenges

Even with meticulous planning, operational hurdles can occur:

Navigational Errors: Mistakes in positioning the cable-laying vessel or malfunctions in navigation equipment can lead to misaligned cables or sections being laid outside the designated route. Utilizing advanced navigation systems and double-checking coordinates are crucial for accurate deployment.

Vessel collision in the offshore construction site

During the installation phase it can be that there is more of one contractor performing works in the offshore site.

Interfaces with Foundations

Turbine Foundations: Inter-array cables are secured to turbine foundations using specialized cable entry systems. These systems prevent water ingress and mechanical damage, ensuring long-term reliability.

Substation Interfaces: At the substation, cables are routed through J-tubes or I-tubes. These tubes guide and protect the cables from external forces, ensuring a secure connection to the substation.

Risks during the installation phase

Installation: The installation process is complex, requiring precise handling to avoid damage.

Risks and Mitigations

Mechanical Damage: Cables and CPS can be damaged by vessel anchors, fishing activities, or seabed movement. Proper routing and protective measures are essential to mitigate these risks.

Environmental Factors: Saltwater, temperature fluctuations, and strong currents can degrade cable materials over time. Selecting robust materials and designs is critical to withstand these conditions.

Statistics Cable Failures

Damage to these cables is a major concern, as it accounts for a substantial portion of insurance claims in the offshore wind industry. For instance, around 50% of claims related to offshore wind farms are due to subsea cable damage, with inter-array cables being particularly vulnerable during installation and operation. he average cost of repairing inter-array cable damage can range from $1.8 million to $12 million, and the downtime for such repairs can be extensive, averaging around 40 days.

CPS Exclusion

The root cause of these issues has been identified as inadequate scour protection, which left the inter-array cables unstabilized. This lack of stabilization allowed the CPS to move across the scour protection rocks, causing abrasion and, in some cases, cable failure. The problem was first detected during an inspection at the Race Bank wind farm and has since been observed in up to ten wind farms, potentially costing up to EUR 403 million in repairs and mitigation measures. To address these issues, Ørsted has implemented a two-phase remediation plan. The first phase involves adding an additional layer of rock protection to stabilize the CPS and prevent further degradation. The second phase includes repairing or replacing damaged cables, scheduled for completion by 2023.

What the underwriters see:

- Track record of the suppliers: mostly related with Property Damage.

- Location of the manufacturing sites: NATCAT?

Important timelines for the insurance team

Contract

For the IAC lot a EPCI will be normally prefered. The EPCI Contractor normally will be a consortium between different specialized companies, for example, a cable supplier, a CPS supplier and a T&I company.

Insurance

During the negotiation with the bidders, there will be normally are interested in these points:

- LEG design clause

- CPS Exclusion

- Extended maintenance

Cut clause

CAR LEG 3 (Construction All Risks - Leg 3): This type of insurance covers physical damage to the insured works due to defects in design, materials, or workmanship, as well as the cost of rectifying such defects. It offers broader coverage than Leg 2. For instance, if a cable is damaged due to a defect, Leg 3 would cover the cable's replacement, removal, and any associated costs, while Leg 2 might only cover consequential damages resulting from the cable failure. The contractor strongly favours Leg 3 coverage, considering it essential for managing the higher risk associated with array cable contracts. he  Leg 3 is generally more cost-effective, as it allows for risk pooling and leverages the insurer's capacity to absorb potential losses.

CAR Leg 2 (Construction All Risks - LEG 2): This insurance type primarily covers consequential damages arising from insured events rather than the cost of rectifying the initial defect. In most of the cases that would lead to not cover the accessing costs, meaning the costs of the vessel.

We, as Employer, would initially propose LEG 2 coverage, citing difficulties in securing LEG 3 coverage in the current insurance market, particularly for cable-related work. We could attribute this difficulty to the insurance market hardening, characterized by insurers' reluctance to offer extensive coverage for high-risk components like cables, due to past loss experiences and a desire to protect their profitability.

Leg 3 DIC (Difference in Conditions): This supplementary insurance policy bridges the coverage gap between Leg 2 and Leg 3. One option for the Contractor is to procuring a Leg 3 DIC policy as a potential solution if the employer cannot provide Leg 3 coverage. However, commercially the costs for this insurance will be passed to the Employer. In addition, this option introduces complexities by involving multiple insurers and potentially increasing administrative overhead.

Indemnity: As an alternative to insurance, the Employer normally can agree to financially compensate the other for specific losses or liabilities. In this context, the contractor could suggest that the Employer could provide an indemnity to cover the risk that would have been covered under a Leg 3 policy. However, the Employer is concerned about assuming the full risk of manufacturing defects, over which they have no direct control.

Extended Maintenance: This insurance covers damages occurring during a specific period after the project's completion. The sources indicate that the standard market practice is to offer extended maintenance for 24 months after the project is taken over. This period often does not align with the contractually agreed-upon warranty period, which can be longer.

Guaranteed Maintenance: This insurance covers the cost of rectifying defects discovered during the warranty period, regardless of when the damage occurred. [26] It provides broader coverage than extended maintenance, which typically focuses on damages occurring during the maintenance period. [26] However, obtaining guaranteed maintenance can be challenging as insurers perceive the risk associated with pre-existing defects as higher and prefer manufacturers to retain this responsibility. [27]

The sources present a complex negotiation landscape where both parties seek to balance risk transfer, cost optimization, and contractual clarity. While various options for addressing the insurance coverage gaps are discussed, no definitive solution emerges within the provided text.

Option 1: Employer Provides CAR Leg 3 Coverage

This option is preferred by contractors as it simplifies the process, reduces costs, and consolidates risk with one insurer. The employer acknowledges the availability of Leg 3 coverage for certain aspects of offshore wind projects and plans to obtain it. However, there are concerns about the availability and cost of Leg 3 coverage for cable-related work due to the insurance market's loss history and risk aversion.

Option 2: Employer Provides CAR Leg 2 Coverage, Contractor Obtains Separate Leg 3 DIC Policy

If the employer cannot secure Leg 3 coverage, the contractor proposes obtaining a separate Leg 3 Difference in Conditions (DIC) policy. This approach, while potentially more costly and complex due to multiple insurers, has seen some recent success. The employer has asked the contractor to quantify the risk differences between Leg 2 and Leg 3 coverage.

Option 3: Employer Provides CAR Leg 2 Coverage and Indemnity

Another alternative if Leg 3 coverage is not feasible is for the employer to provide CAR Leg 2 coverage and an indemnity to cover costs above a certain value. This self-insurance approach raises concerns for the employer about assuming the full risk of manufacturing defects, which they cannot control.

Option 4: Scope-Specific or Sub-Limited Leg 3 Coverage

This option involves obtaining Leg 3 coverage with sublimit for example 30 mio EUR. Challenges include aligning sub-caps with the overall contract liability and ensuring fair risk allocation between parties, particularly in a consortium.

https://www.offshorewind.biz/2021/04/30/orsted-points-at-possible-reason-for-eur-403-million-cable-problem/

https://www.offshorewind.biz/2021/04/30/orsted-points-at-possible-reason-for-eur-403-million-cable-problem/

https://www.4coffshore.com/news/cps-failures-on-%C3%B8rsted92s-offshore-wind-farms-nid24112.html

https://www.imca-int.com/published-articles/imcas-new-code-of-practice-could-save-millions-for-the-offshore-wind-industry-by-preventing-subsea-cable-failure/